# ECOPROSUS SMART AGRICULTURE IoT PROJECT GUIDE

Complete step-by-step reference for ESP32 DHT11, ESP32 Soil Sensor, Raspberry Pi, MQTT, Mosquitto and Node-RED.

# Phase 1: Prepare Raspberry Pi

Update OS and find IP address.
 Step1: 
 sudo apt update
 sudo apt upgrade –y

Step2:
Reboot

### Step3: 
Find Raspberry Pi IP Address use this command    hostname –I
My Raspberry Pi IP is 172.16.0.132

Step4 
Enble static IP : Note ( before  enble need to check this )
1. Prerequisites to Check Before Setting It Up

- Active Network Interface: Check whether you are using Ethernet (eth0) or Wi-Fi (wlan0).

- Router Gateway IP & Subnet Mask: Find your router's IP address (typically 192.168.1.1 or 192.168.0.1) and subnet (usually /24 or 255.255.255.0).

- Current IP Address: Run hostname -I in the terminal to see your Pi's current assigned IP.

- Router DHCP Range: Ensure the static IP you choose is outside your router's active DHCP assignment pool (or reserve it in your router settings) to prevent IP address conflicts with other devices on your network.
Step  5 
After Static IP Configuration successfully 
will get this details 
Raspberry  Pi Ip : 172.16.0.132
Subnet                : /24
Gateway             : 172.16.0.1
DNS                   : 172.16.0.1
Wi-Fi                  : Ecopro
Interface               : wlan0

# Phase 2: Install MQTT Broker

Step1: Install Mosquitto MQTT broker.
for installation use this command 
sudo apt install -y mosquitto mosquitto-clients
Step2 : after install check status use this command 
sudo systemctl status mosquitto it should be Active and  Enble after that move to next step 

Setp 3 : Enable Auto Start 
Use this command 
sudo systemctl enable mosquitto
sudo systemctl start mosquitto


Phase 3: Test MQTT
Verify publish and subscribe.
Setp3 : Test MQTT locally means workign in Rpi or not 
Step1: Open Terminal 1 (The Subscriber):Listening Mode.
Open your first terminal window. This terminal will subscribe to a topic called test/topic and wait for incoming messages.
Run the following command:
mosquitto_sub –h 127.0.0.1 –t “ecoprosus/test”
after running leave terminal 1
Step2: Open Terminal 2 (The Publisher Test Message ):Sending Data.

Open a second terminal window (or a second SSH session/tab). This terminal will publish a test payload to the exact same topic.

Run the following command:
mosquitto_pub –h 127.0.0.1 –t “ecoprosus/test” –m “Hello Ecoprosus”
Expected Result in Termianl 1is 
Hello Ecoprosus

# Phase 4: Device Naming
Before Device Naming have to do this steps 
Step 1: Before Changing the configuratoin, let’s check how Mosquitto is currently listening 
Run belwo command 
sudo ss –lntp | grep 1883
Configure Mosquitto for ESP32
Step2 : First’lets’s create a separate configuration file rather than modifying the main Mosquitto configurtion 
Run this commond 
sudo nano /etc/mosquitto/conf.d/ecoprosus.conf
A blank editor will open
Put this inside : 
listener 1883 0.0.0.0  - (allow MQTT connections on port 1883 from your network interfaces.)
allow_anaymous true – (allows your ESP32 devices to conncect without an MQTT username/ password)
Then save 
Press Ctrl + O
Press Enter 
Press Ctrl + X 

Verify  the configuration fiel 
run this command  
sudo cat /etc/mosquitto/conf.d/ecoprosus.conf

you shoul get this 
listener 1883 0.0.0.0  
allow_anaymous true

Step 3 : Test MQTT from yoru laptop
if your laptop windows 
open PowerShell and run this command 
Test-NetConnection 172.16.0.132 –Port 1883

Will get this 
ComputerName     : 172.16.0.132
RemoteAddress    : 172.16.0.132
RemotePort       : 1883
InterfaceAlias   : Wi-Fi
SourceAddress    : 172.16.0.197
TcpTestSucceeded : True

Actul flow (All are Connected in Sam Network (Wifi))
Laptop ➔Wi-fi ➔Raspberry Pi
                     |
                     |
                     ➔ MQTT1883

Use ECO_DHT01, ECO_SOIL01, ECO_RPI01.

# Phase 5: Install PubSubClient

Install Arduino MQTT library.
This has to install in Arduino IDE 
you will get serial monitor output

# Phase 6: Connect ESP32 WiFi

Configure WiFi credentials. This should be you have update in could pubsubclient code
Follow should be like this 

ESP32 #1

│
Wi-Fi
   │ 
   ▼
Ecopro Network
   │
   ▼
Raspberry Pi
172.16.0.132
    │
    │ MQTT : 1883
     ▼
Mosquitto
    │ 
    ▼
ecoprosus/esp32-01/soil/moisture


For Getting Soil Moisture Data 
Command for Checking data of Soil moisture sensor 
mosquitto_sub -h 127.0.0.1 -t "ecoprosus/esp32-01/soil/moisture"(This is For Moisture)
in terminal you will recevive every 5sec data from ESP32-01 like this (Because ESP32 -01Publishing 50(data ) every 5 seconds )
86
86
86
For Getting DTH11 Data 
Command for Checking data of  DTH11 Temp & Humidity 
mosquitto_sub -h localhost -t "ecoprosus/esp32-02/temperature/humidity" (this for Temperature & Humidity  Data)
If want to see Both data at time use Run this Command 
mosquitto_sub -h localhost -t "ecoprosus/#" -v

in terminal you will recevive every 5sec data from ESP32 like this (Because ESP32 Publishing 50(data ) every 5 seconds )
86
86
86
Today I have completed

# Phase 7: Connect MQTT

Connect to Raspberry Pi broker.
this work testing purpose after completing Phase 8, Phase9 &Phase 10 few commads and test process I have below 
Step 1 : Check Mosquitto Service 
Run Command : sudo systemctl status mosquitto
Step 2 : Verify Broker is Listening on Port 1883
Run Command : sudo netstat -tlnp | grep 1883
Step 3 : See All MQTT Messages(you will see in RPI terminal both sensor data)
Run Command : mosquitto_sub -h localhost -v -t "#"

Step 4 : Monitor Only ESP32-01 Soil Sensor (only soil data in Rpi Terminal)
Run Command : mosquitto_sub -h localhost -v -t "ecoprosus/esp32-01/soil/moisture"
Step 5 : Monitor Only ESP32-02 DHT11 Sensor
Run Command : mosquitto_sub -h localhost -t "ecoprosus/esp32-02/temperature/humidity"
Step 6 :  Test Broker Manually
Run Command : mosquitto_sub -h localhost -t "test/topic"

Step 7 :  Verify ESP32 Can Reach Raspberry Pi
Run Command : mosquitto_sub -h localhost -v -t "#" this commanl use for all 
Run Command :  mosquitto_sub -h localhost -t "ecoprosus/#" -v   (Our project I have set in before #  )

# Phase 8: Publish DHT11

Send temperature and humidity JSON.



This is my JSON format in ESP32-02.
{

"device_id": "ESP32_Node_01",

"online": true,

"timestamp": "2026-09-21T05:36:00Z",

"temperature_c": 27.0,

"humidity_pct": 58.0,

"logs": [

"NTP_SYNC_OK",

"MQTT_CONNECTED"

]

}
Phase 9: Publish Soil Data

Phase 9 : 
Send soil Moisture sensor JSON.
This is my JSON format in ESP32-01.
{

"device_id": "ESP32_Node_01",

"timestamp": "2026-09-21T06:46:00Z",

"readings": {

"soil_moisture_pct": 100.0,

"soil_moisture_raw": 1767

},

"status": {

"sensor": "wet",

"reading_valid": true

}

}

# Phase 10: LED Blink

Blink LED during transmission.
Here I am Using External Red Colour  for  esp32 led to blink, while Data Transmission Double blink led status
GPIO – 18 Pin in ESP32 used for data Transmission 
GPIO -23  Pin in ESP32 used for Acknowledgement.

# Phase 11: ACK

Double blink after acknowledgement.
This one have to one Function Node (in function have to add java script) & MQTT_OUT Node in Node-red and Need work and add led part off the code in ESP32 also 
GPIO23 we are reading led pin

# Phase 12: Install Node-RED

Install Node-RED dashboard platform.
Before instaling  that we have to update
sudo apt update 
sudo apt upgrage –y
Run This Command to Install 
https://raw.githubusercontent.com/node-red/linux-installers/master/deb/update-nodejs-and-nodered

Verify Node Red  : Run this Command 
node-red –version 

Check the status of Nod-Red : Run this Command 
sudo systemctl status nodered

Start Node-Red : Run this Command 
sudo systemctl start nodered

enable Node-Red : Run this Command 
sudo systemctl enable nodered

Phase 13: Create Dashboard
Before this you open Node-Red web browser in chorme 

Step 1: you do in web
Step 2 — Configure MQTT
In Node-RED on your PC:
Drag mqtt in onto the workspace.
Double-click it.
For Server, click the pencil ✏️.
Enter:below details 
| Setting    | Value                                     |

| ---------- | ----------------------------------------- |

| **Server** | `localhost:1883`                          |

| **Action** | Subscribe to single topic                 |

| **Topic**  | `ecoprosus/esp32-02/temperature/humidity` |

| **QoS**    | `0`                                       |

| **Output** | `auto-detect`                             |

| **Name**   | `ESP32-02 Temp Humidity`                  |

Step 3 :

Drag Debug  onto the workspace
Connect output of mqtt to input of Debug like this we have do

# Phase 14: Dashboard Layout

Create Ecoprosus monitoring screen.
I have already Created Layout, Need to create group, page,ect in Node-Red

# Phase 15: Data Storage

Use SQLite/InfluxDB.

# Phase 16: Cloud Upgrade

Move to AWS IoT in future.

PHASE 1: Raspberry Pi

│

├── 1.1 Update OS

│

├── 1.2 Find IP

│

└── 1.3 Static IP  ← YOU ARE HERE

│

▼

PHASE 2: Mosquitto MQTT

│

▼

PHASE 3: MQTT Test

│

▼

ESP32 + DHT11 + FC-28

│

▼

MQTT

│

▼

Raspberry Pi

│

▼

Node-RED

│

▼

Ecoprosus Dashboard